We need a fundamental change how we are doing the scan .  we need to be very  determinalistic  in the approach,     So all 64 bits has it own time slice;

the biggest star of this operation is going to be the shift register and how we do that.   The Update  Shift Reg routine need to be a tight as possible because it the basic timing block.  We need 4 updates to generate each pulse

Lamp_Data – array  4 columns and 8 rows ( bit array)

Switch_Data – array 4 columns and 8 rows ( bit array)

Shift Reg Data array - 16 bits (only 12 being used)

For Row= 1 to 8

For Column 1 to 9

If Current column <  4      // (lamps)

Read data array for lamps Lamp Data(row,col)

IF Lamp Data(row,col) =1 then pulse lamp on  If not then continue to check next element

- Transfer current shift_registeter Data to set column active

- Set current row  =on ( update shift reg )

- Delay for pulse width 2-5us // lamp Pulse width adjustment

- set row pin off   // set all row pins off here (update shift reg)

- Transfer current shift_registeter Data to set all columns low  // AND bits to turn o

IF current column < or = 4 then

- Transfer current shift_registeter Data to set column active

- Set current row pin =

- Delay for pulse width 2-5us // Pulse width adjustment

- Read Input data  and store //

- set row pin off   // set all row pins off here

- Transfer current shift_registeter Data to set all columns low  // AND bits to turn off